❶ Internal volume: 20000L-33000L;

❷ Temperature control range: 20～45℃; temperature fluctuation: ±0.5℃; temperature deviation: ±1.0℃;

❸ Humidity control range: 20～80%RH; Humidity control fluctuation: ±2.0%RH; Humidity deviation: ±3.0%RH;

❹ Temperature and humidity controller: large screen color touch screen controller, with frequency output value, with three levels of authority and audit trail;

❺ Humidity sensor: imported high-precision capacitive humidity sensor, direct detection, no maintenance;

❻ Refrigeration compressor: imported hermetic frequency conversion compressor (indoor or outdoor machine can be, a backup);

❼ Data printing: can print the model and number of the equipment, can print the temperature and humidity values and curves at the same time;

❽ Data storage: can store more than 5 years of temperature and humidity values;

❾ On-site alarm: sound and light alarm for temperature and humidity deviation on-site, and independent over-temperature alarm;

❿ Remote alarm: remote cell phone SMS alarm (with power failure alarm), multiple devices can share a cell phone card;

⓫ Chamber material: the inner liner material for stainless steel 304, shell for polyurethane foam insulation board;

⓬ Chamber door structure: with hollow glass observation window; with door lock, support the internal open door lock;

⓭ Fivefold protection: configure double insurance anti-dry burning protection system; configure independent over-temperature protection system, can automatically cut off the power supply;

⓮ Safety devices: compressor overheating protection, compressor overpressure protection, compressor overload protection and fan overheating protection.

⓯ Installation power: 380V.

⓰ Other configurations: see specific program for details.

| Model | Temperature Range ( ℃ ) | Humidity Range (RH) | Capacity (L) | Interior Dimensions W×D×H(mm)​ | External Dimensions W×D×H(mm) | Power (kW) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Labonce-20000GS-FC | 20 ~ 45 | 20~80% | 20000 | 2200×4100×2200 | 2400×4300×2400 | 6.0 | Customizable Humidity ： 40 ~ 80%RH |
| Labonce-33000GS-FC | 20 ~ 45 | 20~80% | 33000 | 2950×5200×2200 | 3150×5400×2400 | 6.5 |  |