❶ Internal Volume: 300L-600L
❷ Temperature Control Range: 15-65°C, Temperature Fluctuation: ±0.5°C, Temperature Deviation: ±1.0°C (without light)
❸ Humidity Control Range: 20-95%RH, Humidity Fluctuation: ±2.0%RH, Humidity Deviation: ±3.0%RH
❹ Temperature & Humidity Controller: Programmable color touch screen controller with variable frequency output, program control, and three-level access control
❺ Humidity Sensors: Two sets of imported high-precision capacitive humidity sensors for direct detection, maintenance-free
❻ Refrigeration Compressors: Two sets of imported variable frequency hermetically sealed compressors with long lifespan and low noise
❼ Data Printing: Two printing systems that can print the equipment model and serial number, as well as temperature/humidity values and curves simultaneously
❽ Data Storage: Two storage systems capable of storing temperature/humidity data for over 5 years
❾ On-site Alarm: Audible alarm for temperature/humidity deviations and independent over-temperature alarm
❿ Remote Alarm: Remote SMS alarm (including power failure alarm), multiple devices can share one SIM card
⓫ Chamber Material: Interior made of mirror-finish stainless steel 304, exterior made of high-quality powder-coated steel plate

⓬ Dual Protection: Dual dry-burn prevention system and independent over-temperature protection system with automatic power cutoff
⓭ Safety Devices: Compressor overheat protection, compressor overpressure protection, compressor overload protection, and fan overheat protection
⓮ Power Supply: AC 220V±10%, 50Hz
⓯ Operating Environment: +5-35°C (recommended to control within 30°C, limit condition should be controlled within 26°C)

| Model | Temperature Range( ℃ ) | Humidity Range(RH) | Capacity (L) | Interior Dimensions W×D×H(mm) | External Dimensions W×D×H(mm) | Power (kW) | Shelf (Standard) | Chamber Structure | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Labonce-320CGS-FC | 15~65 | 20~95% | 150 | 680×500×460 | 860×840×1880 | 2.5 | 2/5 | UP A | Temp deviation：＜ ±1.0℃ 2%RH Humidity deviation：＜ ±3%RH A /B (Temperature + Humidity) |
|  |  |  | 150 | 680×500×460 |  |  | 2/5 | Down B |  |
| Labonce-520CGS-FC | 15~65 | 20~95% | 250 | 600×500×830 | 1610×870×1650 | 3.0 | 3/7 | Left A |  |
|  |  |  | 250 | 600×500×830 |  |  | 3/7 | Right B |  |
| Labonce-620CGS-FC | 15~65 | 20~95% | 300 | 600×500×1000 | 1610×870×1820 | 3.2 | 3/9 | Left A |  |
|  |  |  | 300 | 600×500×1000 |  |  | 3/9 | Right B |  |
| Remarks 2 | 1）CGS-FC Series: Chamber A is equipped with three functions: temperature, visible light, and near ultraviolet. Illuminance measurement comes standard with visible light and near ultraviolet sensors, and both visible light and near ultraviolet values are automatically printed and stored. 2）Visible light range: 100 ~ 6000Lux, illuminance deviation: 4500 ± 500Lux, the total illuminance of the illumination test shall not be less than 1.2×106 Lux·hr; near ultraviolet range: 0.84 ~ 1.0 w/m2 , the total illuminance of the near ultraviolet shall not be less than 200w·hr/m2 .. When lighting is off, temperature and humidity speciffcations are the same as GS-FC. 3）Temperature ffuctuation: < ±0.5°C ； Temperature deviation: < ±1.0°C (when lighting is off) ； Humidity ffuctuation: < ±2% RH ； Humidity deviation: < ±3% RH (when lighting is off). |  |  |  |  |  |  |  |  |