❶ Chamber capacity: 430L-930L
❷ Temperature control: Range 15-65°C, Stability ±0.5°C, Uniformity ±1.0°C (without illumination)
❸ Humidity control: Range 20-95%RH, Stability ±2.0%RH, Uniformity ±3.0%RH (without illumination)
❹ Visible light: 100-6,000 Lux (minimum total illumination: 1.2×10⁶ Lux·hr)
❺ Near-UV: 0.84-1 W/m² (minimum total energy: 200 W·hr/m²)
❻ UV wavelength: 320-400 nm
❼ Integrated touchscreen control (temperature/humidity/visible light/UV)
❽ Simultaneous parameter printing (temperature/humidity/light/UV)
❾ Large color touchscreen with programmable settings and three-level access control
❿ Imported capacitive humidity sensors (maintenance-free)
⓫ Three imported inverter compressors (long lifespan, low noise)
⓬ Data storage: 5+ years with real-time curve recording
⓭ Alarm system: Audible/visual alarms, remote SMS alarm (including power failure)
⓮ Construction: 304 stainless steel interior (mirror finish), powder-coated steel exterior
⓯ Safety: Dual protection systems, comprehensive compressor protections
⓰ Power: AC 220V±10%, 50Hz
⓱ Operating conditions: Ambient +5-35°C (Recommended ≤30°C, Maximum ≤26°C)

| ​Model | Temperature Range( ℃ ) | Humidity Range (RH) | Capacity (L) | Interior Dimensions W×D×H(mm) | External Dimensions W×D×H(mm) | Power (kW) | Shelf (Standard) | Chamber Structure |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Labonce-430GS-FC | 15～65 | 20～ 95% | 100 | 480×420×500 | 1370×780×1930 | 3.6 | 1/4 | A (Temperature + Humidity + Visible Light + Near Ultraviolet) B (Temperature + Humidity) |
|  |  |  | 100 | 480×420×500 |  |  | 2/4 |  |
|  |  |  | 230 | 480×420×1100 |  |  | 4/8 |  |
| Labonce-930GS-FC | 15～65 | 20～ 95% | 220 | 600×700×520 | 1570×1060×1960 | 4.0 | 1/6 |  |
|  |  |  | 220 | 600×700×520 |  |  | 2/6 |  |
|  |  |  | 480 | 600×700×1140 |  |  | 4/8 |  |
| Labonce-430CGS-FC | 15～65 | N/A | 100 | 480×420×500 | 1370×780×1930 | 3.6 | 1/4 | A (Temperature + Visible Light + Near Ultraviolet; when lighting is on: temperature deviation on the same level is ±2.0°C) B/C (Temperature + Humidity) |
|  |  | 20～ 95% | 100 | 480×420×500 |  |  | 2/4 |  |
|  |  | 20～ 95% | 230 | 480×420×1100 |  |  | 4/8 |  |
| Labonce-930CGS-FC | 15～65 | N/A | 220 | 600×700×520 | 1570×1060×1960 | 4.0 | 1/6 |  |
|  |  | 20～ 95% | 220 | 600×700×520 |  |  | 2/6 |  |
|  |  | 20～ 95% | 480 | 600×700×1140 |  |  | 4/8 |  |
| Remarks 2 | Labonce CGS-FC Series' Chamber A integrates temperature, visible light (100-6000Lux, ≥1.2×10⁶ Lux·hr) and near-UV (0.84-1.0W/m², ≥200W·hr/m², 320-400nm) controls with automatic monitoring/printing of both light parameters, while maintaining GS-FC's temperature/humidity performance in non-illumination mod. |  |  |  |  |  |  |  |