❶ Internal Volume: 700L;
❷ Temperature Range: 0～70℃; Temperature Fluctuation: <±0.2℃; Temperature Deviation: <±0.5℃;
❸ Humidity Range: 10～80%RH; Humidity Fluctuation: <±1.5%RH; Humidity Deviation: <±2.0%RH;
❹ Temperature & Humidity Controller: Large programmable color touchscreen controller (7 inches or above), with three-level access control and audit trail functionality;
❺ Humidity Sensor: Imported high-temperature-resistant capacitive humidity sensor, direct detection, maintenance-free;
❻ Refrigeration Compressor: Imported fully enclosed inverter compressor, long lifespan,low noise;
❼ Data Storage: Electronic data storage function, retention for over 5 years;
❽ Data Interface: Supports MODBUS-RTU communication protocol;
❾ On-Site Alarm: Audible alarm for temperature/humidity deviation, independent over temperature alarm;
❿ Remote Alarm: Remote SMS alert (including power failure alarm), multiple devices can share one SIM card;
⓫ Chamber Material: Interior and exterior constructed with 304 stainless steel for superior corrosion resistance;
⓬ Dual-Door Structure: Inner tempered glass door for easy observation; outer solid door blocks external light interference;
⓭ Five-Level Protection: Dual dry-burn protection system; independent over-temperature protection with automatic power cutoff;
⓮ Safety Features: Compressor overheat protection, overpressure protection, overload protection, and fan overheat protection;
⓯ Power Supply: AC 220V±10%, 50Hz;
⓰ Operating Environment: +5～35℃ (recommended below 30℃; for extreme conditions, maintain below 25℃)

| Model​ | Temperature range (℃) | Humidity range (RH) | Volume (L) | Inner dimensions W×D×H(mm) | Overall dimensions W×D×H(mm) | Power (KW) | Shelf (standard) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Labonce-720FS | 0～ 70 | 10～80% | 700 | 975×575×1250 | 1280×880×1940 | 2.5 | 4/8 |