❶ Internal volume: 8000-80000L;

❷ Temperature control range: 20～35℃; temperature fluctuation:±0.5℃; temperature deviation: ±1.0℃;

❸ Temperature and humidity controller: large screen color touch screen controller, program control, with three levels of authority;

❹ Refrigeration compressor: imported hermetic frequency conversion compressor (indoor or outdoor machine can be, a backup);

❺ Data printing: can print the model and number of the equipment, can print the temperature value and curve at the same time;

❻ Data storage: can store temperature values for more than 5 years;

❼ On-site alarm: on-site temperature and humidity deviation sound and light alarm, independent over-temperature alarm;

❽ Remote alarm: remote cell phone text message alarm (with power failure alarm), multiple shared;

❾ Chamber material: liner material for mirror stainless steel 304, shell for polyurethane foam insulation board;

❿ Insulation material: polyurethane foam, B1 fireproof grade, thickness 75~100mm.

⓫ Chamber door structure: with hollow glass observation window; with door lock, support internal open door lock;

⓬ Double protection: equipped with independent over-temperature protection system and leakage protection, can automatically cut off the power supply;

⓭ Safety devices: compressor overheating protection, compressor over pressure protection, compressor overload protection and fan overheating protection.

⓮ Installation power: 220V or 380V.

⓯ Other configurations: See specific program for details.

| Model | Temp range (℃ ) | Volume (m3) | Inner size W×D×H (mm) | Outside size W×D×H (mm) | Power(kW) |
| --- | --- | --- | --- | --- | --- |
| Labonce-8000CT-FC | 20 ~50 | 8 | 1900×2200×2000 | 2050×3150×2250 | 4.0 |
| Labonce-15000CT-FC | 20 ~50 | 15 | 1900×4000×2000 | 2050×4950×2250 | 4.5 |
| Labonce-20000CT-FC | 20 ~50 | 20 | 2200×4100×2200 | 2400×4300×2400 | 4.5 |
| Labonce-33000CT-FC | 20 ~50 | 33 | 2950×5200×2200 | 3150×5400×2400 | 5.5 |
| Labonce-40000CT-FC | 20 ~50 | 40 | 3600×5200×2200 | 3800×5400×2400 | 6.0 |
| Labonce-60000CT-FC | 20 ~50 | 60 | 4400×6400×2200 | 4600×6600×2400 | 7.0 |
| Labonce-80000CT-FC | 20 ~50 | 80 | 5800×6400×2200 | 6000×6600×2400 | 8.0 |